Above Ground Pool Guide

Volume:

12’ diameter, 36” deep – 2,500 (approximately)

15’ diameter, 36” deep – 3,500 (approximately)

Pump should be running for approximately 5 hours a day, but not when in use.

Chemical Levels

Ph : 			7.2 – 7.6

Chlorine Level: 			3-5PPM

To DECREASE PH use DRY ACID

12’ diameter: add 150 grams daily until correct level is reached.

15’ diameter: add 210 grams daily until correct level is reached.

To INCREASE PH use DRY ALKALI

12’ diameter: add 150 grams daily until correct level is reached.

15’ diameter: add 210 grams daily until correct level is reached.

To ADD  1ppm of Chlorine

12’ diameter: add 15 grams

15’ diameter: add 21 grams

To Lower Chlorine Level

Remove part of the water and replace it with tap water. Chlorine will reduce in time, in extreme cases use chlorine neutraliser – sodium thiosulphate.

With these chemicals, add warm water and dissolve. Add evenly over the surface of the water. Do not add whilst pool is in use.

Longlife Algaecide is recommended to ensure an algae free summer. Add as per manufacturers instructions at the start of the season.

If you have any problems, please do not hesitate to contact us at any time.

Call 01747 839608 for more advice