Underwater Lighting Information

Calculating the number of  Lights per pool

Sealed Beam 300W 12v Par 56 lights

Commercial Pools

Due to the size of commercial pools, in particular the width, we recommend that lights are installed at 3 metres intervals around the pool perimeter. The beam width of the Par 56 bulbs is 3 metres, so by this spacing the maximum coverage and illumination can be achieved. In pools that will be used for racing we do not recommend lights at either end of the pool.

Domestic Pools

A	For indoor pools where the lights will be used in conjunction with overhead or partial overhead lighting the recommendation is one light for the first  38m² (400 ft².) and one for every 24m² (250 ft².) thereafter.

B	For indoor pools where the pool lights will be used for effect: the recommendation is one light for the first 24m² (250 ft².)  and one for every 24m² (250 ft².) thereafter.

C	For outdoor pools where pool lights will be used in conjunction with area lighting or general lighting  the recommendation is one light for the first   30m² (320 ft².) and one for every 19m² (200 ft².) thereafter.

D	For outdoor pools where the pool lights will be used for effect the recommendation is one light for the first  24m² (250 ft².)  and one for every 19m² (200 ft².) thereafter.

LED lights

TK Lights Par 56

Recommendation is for 1 light per 20m2 of water surface, in especially deep pools 1 light per 25m3 of water volume would be sufficient.

Mini light

Recommendation is for 1 light per 5m2 water surface, in deep pools 1 light per 10m3 of water volume would be sufficient.

Fibre Optics

Spots

For good illumination approx 8 strands per m2 is required.

Perimeter

The basic concept behind the perimeter lighting is to enhance the pool edge, it provides no light to the pool interior.